Output Channel Error Handling Algorithm CEH.GD.Auto

(Formalized Specification)

Pi::CEH.GD.Auto

{SYNOPSIS}

The algorithm is used to automatically keep the output connection open. Supports "graceful degradation" in combination with the algorithm RUP.GD for communication ring integrity checkup.

The algorithm monitors the state of the output channel to the immediate neighbor and is responsible for its opening. Starts just after process creation as well as at the process output disconnection with its immediate neighbor.

{ASSUMPTIONS}

The CEH.GD.Auto algorithm is valid if the following conditions are met:

- The distributed system is synchronous.

- The type of process failures is “fail-stop”; however, is allowed the process auto reparation before it is finally marking as faulty.

ALGORITHM 1: Declarative Part of Pi::CEH.GD.Auto

{SYSTEM CONSTANTS}

Int MAX_CEH_PERIOD		// period between reconnection attempts

Int MAX_CEH_ERR		// maximum reconnection attempts

Int MIN_K			// min number of faultless processes (max degradation), 1 ≤ MIN_K

PId i				// process Pi identifier

PId j				// default neighbor process Pj

Bool AutoList			// <true> if ListPIds is to be filled during RUP.GD.Auto

// <false> if ListPIds is known in advance (RUP used)

{MESSAGES}

{SET OF STATES}

<State> := {INIT, CLOSED, OPENED, FAULTY}

{INTERNAL STATE SPACE}

State state			// current process CEH state

Int ErrorCounter			// error counter

Timer TimerCEH			// timer

List ListPIds			// list of process identifiers

PId PIdNext			// current neighbor process identifier

ALGORITHM 2: Event Handlers of Pi::CEH.GD.Auto

OnInit:

state := INIT

ErrorCounter := 0

TimerCEH.Interval := MAX_CEH_PERIOD

ListPIds.Set()

If AutoList = true

ListPIds.Clear()

PIdNext := j

Else

ListPIds.Set()

PIdNext := ListPIds.PopFront()

End If

OnShow:

ErrorCounter := 0

ChannelOut.Open()

OnOutputConnect:

state := OPENED

ErrorCounter := 0

{Start Ring Check Up Algorithm}

OnOutputDisconnect:

state := CLOSED

ErrorCounter := 0

TimerCEH.Start()

OnOutputError:

state := CLOSED

ErrorCounter := ErrorCounter + 1

If ErrorCounter < MAX_CEH_ERR

TimerCEH.Start()

Else

If ((AutoList = false) ∪ (RUP::state = UP)) ∩ (ListPids.Size() ≥ MIN_K)

PIdNext := ListPIds.PopFront()

ErrorCounter := 0

TimerCEH.Start()

Else

{UNRECOVERABLE FAILURE}

End If

End If

OnTimer:

TimerCEH.Stop()

ChannelOut.Open()